Content Guidance

The file must contain information about the current project likely to be needed to ensure health and safety during any subsequent work, such as maintenance, cleaning, refurbishment, or demolition.

If Principal Designer

“The principal designer must prepare the health & safety file.”

But this is primarily a coordination role and the PD must expect the cooperation of the rest of the project team including the Principal Contractor and the Client team. This is to ensure that the structure and content are agreed early and who should provide the relevant information and to what programme. Progress of the file should commence from start on site and be checked regularly at Design Team and Progress meetings using this tracker or other suitable means.

If Designer

Where it is not possible to eliminate health and safety risks when preparing or modifying designs, designers must ensure appropriate,

information is included in the health and safety file about the reasonably practicable steps they have taken to reduce or control those risks.

When preparing the health and safety file, information on the following should be considered for inclusion: -

- a brief description of the work carried out.

- any hazards that have not been eliminated through the   design and construction processes, and how they have been addressed (e.g., surveys or other information concerning asbestos or contaminated land).

- key structural principles (e.g., bracing, sources of substantial stored energy –including pre- or post-tensioned members) and safe working loads for floors and roofs,

- hazardous materials used (e.g., lead paints and special coatings);

- information regarding the removal or dismantling of installed plant and equipment (e.g., any special arrangements for lifting such equipment);

- health and safety information about equipment provided for cleaning or maintaining the structure,

- the nature, location and markings of significant services, including underground cables; gas supply equipment; fire-fighting services etc.

- information and as-built drawings of the building, its plant and equipment (e.g., the means of safe access to and from service voids and fire doors).

There should be enough detail to allow the likely risks to be identified and addressed by those carrying out the work. However, the level of detail should be proportionate to the risks.

Note: -

The file should not include things that will be of no help when planning future construction work such as pre-construction information, the construction phase plan, contractual documents, safety method statements etc. Information must be in a convenient form, clear, concise and easily understandable.

| 1.0 | Brief description of the work carried out | 1.01 | Address of project | DRM 1 | PD/Client |
| --- | --- | --- | --- | --- | --- |
|  |  | 1.02 | Location plan | DRM 1 | PD/Client |
|  |  | 1.03 | Description of the project | DRM 1 | PD/Client |
|  |  | 1.04 | Client/ Ownership of Project/asset |  | PD/Client |
|  |  | 1.05 | Principal Designer, Consultants, Principal Contractor | DRM 1 & 1a | PD/Client |
|  |  | 1.06 | Specialist Sub-Contractors | O&MM | PC |
|  |  | 1.07 | Operational Requirements and Constraints | DRM 1 | PD/Client |
|  |  | 1.08 | Fire Safety Strategy (pre-existing building) | DRM2 | Client |
| 2.0 | Hazards that have not been eliminated through the design and construction processes, and how they have been addressed (e.g., surveys or other information concerning asbestos, lead paint or contaminated land that remains) | 2.01 | HAZARD Information and COSHH Sheets – | DRM 3 & 4 | PD & PC |
|  |  | 2.02 | Details of the General Construction methods and materials used that present significant residual hazards respecting cleaning, maintenance and/or demolition. |  |  |
| 3.0 | Key structural principles | 3.01 | Safe working loads for floors and roofs. | DRM 3 & 4 | Struct. Eng |
|  |  | 3.02 | Bracing, sources of substantial stored energy – including pre- or post-tensioned members, cantilevers, scaffold support loads, temporary works support capacity, etc. | DRM 3 & 4 | Struct. Eng |
| 4.0 | Hazardous materials used (if any) (e.g., epoxy paints and special coatings); | 4.01 | Maintenance Instructions for Project Fabric, including Access | DRM 3 & O & M | All |
| 5.0 | Information regarding the removal or dismantling of installed plant and equipment. (e.g., any special arrangements for lifting such equipment) | 5.01 | General description of essential services and other installations showing access for removal intact or by dismantling and removal routes horizontally and vertically | DRM 3 & 4 | Services Eng |
| 6.0 | Health and safety information about equipment provided for cleaning or maintaining the structure; | 6.01 | HAZARD Information and COSHH Sheets – General Services instructions concerning significant residual hazards for cleaning, maintaining, or dismantling, Access to Project Equipment and any requirement for a Permit to Work | DRM 3 & 4 & O&M | PD & PC |
|  |  | 6.02 | User Guide Notes | O & MM | C&A Consultant |
| 7.0 | Nature, location, and markings of significant services, including underground cables; gas supply equipment; fire-fighting services etc.; | 7.01 | Updated statutory authority and other project services record documents, inside and outside the building | Project Drawings | Services Consultant |
| 8.0 | Information and as-built drawings of the building, its plant and equipment. (ie. All “final contract issue” or “as-built” information). | 8.01 | Building surveys or reports updated to reflect new structure or equipment. Asbestos Register- updated | O & MM | All Project Team to provide relevant information |
|  |  | 8.02 | General Arrangement drawings relating to the “as-built” conditions. | H & S File | PD & PC to collaborate |
|  |  | 8.03 | Architectural/Lead Designer’s other drawings etc. | O & MM | PD |
|  |  | 8.04 | Structural /Civil Engineers’ drawings etc. | O & MM | Struct. Eng |
|  |  | 8.05 | Landscape designs, etc. | O & MM | Landscape Designer |
|  |  | 8.06 | Specialist Fabric Consultant/Contractor’s designs e.g., Facades, roofs | O & MM |  |
|  |  | 8.07 | Services Engineering designs; (Plumbing/ Public Health) | O & MM | Services Designer |
|  |  | 8.08 | Services Engineering designs; (Mechanical) | O & MM | Services Designer |
|  |  | 8.09 | Services Engineering designs; (Electrical) | O & MM | Services Designer |
|  |  | 8.10 | Services Engineering designs; (Specialist) | O & MM | Services Designer |
|  |  | 8.11 | Fire protection drawings, compartmentation, fire stopping, passive and active systems. | H & S File | Passive Fire Consultant/ Fire Eng |
|  |  | 8.12 | Fire Strategy /Means of escape drawings | H & S File | Fire Eng/PD |
|  |  | 8.14 | Services isolation points and fixed facilities for fire fighting. | H & S File | Services Designer |
|  |  | 8.15 | Emergency lighting | H & S File | Services Designer |